🌐 The 5 Layers of Networking (Easy Explanation)

Think of networking like delivering a package.
Each layer has a different job.

🔵 1. Physical Layer (Layer 1)

What it is:
The actual hardware — cables, connectors, wires, Wi-Fi signals.

What it does:

- Sends electrical signals, light pulses, or radio waves

- Defines cable types (Ethernet cable, fiber, etc.)

- Defines connectors and voltages

Analogy:
🚚 The delivery truck and the roads it drives on.

🟢 2. Data Link Layer (Layer 2)

Other name: Network Access Layer

What it does:

- Interprets signals into frames

- Responsible for communication within the same network

- Uses MAC addresses

- Most common protocol: Ethernet (or Wi-Fi)

Analogy:
🛣️ How the truck gets from one street to the next, intersection by intersection.

🟠 3. Network Layer (Layer 3)

What it does:

- Moves data between different networks

- Uses IP addresses

- Uses routers to find the best path

- Most common protocol: IP (Internet Protocol)

Analogy:
🗺️ The GPS map that chooses which roads the truck must take from city A to city B.

🟡 4. Transport Layer (Layer 4)

What it does:

- Makes sure data goes to the correct application

- Uses port numbers

- Uses two main protocols:

  - TCP → reliable delivery

  - UDP → fast but no guarantee

Analogy:
🚪 The delivery driver knocking on the right door in the house and handing the package to the correct person.

🔴 5. Application Layer (Layer 5)

What it does:
Provides the protocols used by actual apps like:

- HTTP (web browsing)

- SMTP (email sending)

- IMAP/POP (email receiving)

- FTP (file transfer)

Analogy:
📦 The actual contents of the package — the item you wanted.

⭐ Super Simple Summary Table

| Layer | What It Handles | Example | Analogy |
| --- | --- | --- | --- |
| 5.Application | Apps & services | HTTP, SMTP | The package itself |
| 4. Transport | Sends data to correct app | TCP, UDP | Delivery driver knocking |
| 3. Network | Routing between networks | IP | Choosing best road route |
| 2. Data Link | Communication inside same network | Ethernet, Wi-Fi | Street-to-street travel |
| 1. Physical | Hardware signals | Cables, radio | The road and the truck |

🧠 Why layers are useful

- Helps understand how data travels

- Makes troubleshooting easier

- Let’s hardware and software from different companies work together

🔌 1. What Are Network Cables?

Cables connect computers, switches, and routers so data can travel between them.

There are two major types:

- Copper cables

- Fiber optic cables

🟠 2. Copper Cables (most common)

✔ What they are:

- Made of copper wires inside plastic

- Carry electrical signals (changes in voltage) to represent 1s and 0s

- Used in most homes and offices

✔ Types:

| Type | Speed | Notes |
| --- | --- | --- |
| Cat 5 | Older, slow | Mostly replaced |
| Cat 5e | Faster | Less crosstalk → fewer errors |
| Cat 6 | Even faster | Higher quality, more expensive |

✔ What is Crosstalk?

- When a signal on one wire leaks into another wire

- Causes errors (data gets misunderstood)

- Cat 5e and Cat 6 reduce crosstalk → better performance

✔ Pros of Copper:

- Cheap

- Flexible

- Easy to install

✔ Cons:

- Limited speed compared to fiber

- Limited distance

- Sensitive to electrical interference

🔵 3. Fiber Optic Cables (faster, long-distance)

✔ What they are:

- Thin glass tubes (as thin as a hair!)

- Send light pulses, not electricity

- Light = 1 or 0 depending on the beam pattern

✔ Pros of Fiber:

- Much faster speeds

- Can go much longer distances without losing signal

- Not affected by electrical noise

✔ Cons:

- More expensive

- More fragile

- Mostly used in data centers, ISP networks, backbone links — not usually in homes

🌟 Simple Comparison: Copper vs Fiber

| Feature | Copper Cable | Fiber Optic |
| --- | --- | --- |
| Signal | Electricity | Light |
| Speed | Good | Very fast |
| Distance | Short | Very long |
| Cost | Cheap | Expensive |
| Interference | Sensitive | Immune |
| Common Use | Homes/offices | Data centers, long links |

🧠 Quick Memory Trick

Copper = cheap and short
Fiber = fast and far

🌐 Why do we need network devices?

Cables connect two computers directly (point-to-point).
But in the real world, we need many computers to talk to each other.

So we use devices like:

- Hubs

- Switches
(and later: routers)

Let’s start with the simplest ones.

🔌 1. Hub (Old, simple, mostly NOT used today)

- A hub works at Layer 1 (Physical layer).

- It connects multiple devices, but it is dumb — it does NOT understand who data is meant for.

✔ What happens when a device sends data through a hub?

The hub copies the data to ALL connected devices.

Every computer receives the data.
Each device must check:
“Is this for me?” → If yes, accept;
If not → ignore.

❌ Problems with hubs:

- Creates a collision domain
(only ONE device can talk at a time)

- If two send at the same time → collision

- Devices must wait and retry → slow network

- Lots of unnecessary noise and traffic

🏁 Result:

Hubs became rare because they create too many collisions and slow networks.

🔄 2. Network Switch (Modern, smart, commonly used)

- A switch works at Layer 2 (Data Link layer).

- It understands MAC addresses, so it knows exactly where to send data.

✔ What happens when a device sends data through a switch?

- Switch reads the destination MAC address

- Switch sends the frame only to that one device

- Other devices do NOT receive the data

✔ Why switches are better than hubs:

- No unnecessary traffic

- Fewer or zero collisions

- Much faster performance

- Multiple devices can talk at the same time

🧠 Switch = smart hub

It was originally called a switching hub, but it is far more intelligent.

⭐ Simple comparison

| Feature | Hub | Switch |
| --- | --- | --- |
| Layer | Layer 1 | Layer 2 |
| Smart? | ❌ No | ✔ Yes |
| Sends data to | All devices | Correct device only |
| Collision domain | One big one | Reduced / none |
| Speed | Slow | Fast |
| Used today? | Rare | Very common |

📦 Easy analogy

- Hub → megaphone
You shout, and everyone hears you.

- Switch → private phone call
Only the intended person receives the message.

🌐 Why do we need routers?

- Hubs and switches connect devices inside one network (LAN).
Example: all devices inside your home or office.

But what if you want to send data to:

- Another building

- Another city

- Another country

- A website on the Internet?

👉 You need a router.

Routers connect different networks together.

🔵 What is a Router?

A router is a Layer 3 (Network layer) device.

It can:

- Read IP addresses

- Decide where to send data between networks

- Forward data to the correct next router on the path

Switch reads MAC → Router reads IP.

🏠 Home Router (Simple Router)

This is the Wi-Fi router you have at home.

Its job is:

- Take data from your home network (LAN)

- Forward it to your ISP

- Get the response from the ISP

- Send it back to your devices

It usually has:

- Small routing table

- Simple decisions (mostly “send to ISP”)

🏢 Core Routers (Internet Routers)

These belong to big ISPs (Airtel, Jio, AT&T, etc.)

They are:

- Very powerful

- Handle millions of packets per second

- Know how to reach every network in the world

They form the backbone of the Internet.

These routers talk to each other using:

🔄 BGP – Border Gateway Protocol

- BGP allows routers to share information about
the best path to reach each network globally.

- Helps build a map of the Internet.

🚀 What happens when you load a webpage?

- Your laptop → sends data to home router

- Home router → sends to your ISP router

- ISP router → sends to another router → next → next → next

- Your data may pass through dozens of routers

- Eventually reaches the website's server

- Server sends data back along the reverse path

Routers work like global traffic guides, making sure your data finds the correct destination.

📦 Simple Analogy

- Switch = inside a building → decides which room to send the message to

- Router = city post office → decides which city or country to send the letter to

- Core router = international airport hub → handles global mail routes

⭐ Summary Table

| Device | Layer | What it does | Where used |
| --- | --- | --- | --- |
| Hub | Layer 1 | Broadcasts data to all | Rarely used |
| Switch | Layer 2 | Sends data to correct device in LAN | Offices, homes |
| Router | Layer 3 | Sends data between networks | Homes, ISPs |
| Core Router | Layer 3 | Routes Internet traffic globally | ISP backbone |

🌐 Why talk about Servers and Clients?

All the devices you learned about (switches, routers, hubs) help computers communicate.
But what are the computers actually doing?

Usually:

- One computer asks for data

- Another computer provides the data

This creates the client–server relationship.

🖥️ What is a Server?

A server is anything that provides data or services.

Examples:

- A web server sends webpages

- A mail server sends/receives emails

- A file server shares files

- A DNS server gives IP addresses

You can think of a server as a giver.

💻 What is a Client?

A client is anything that requests or receives data from a server.

Examples:

- Your web browser (Chrome, Edge) → client

- Your email app → client

- A phone app → client

A client is a requester.

🔄 Servers and Clients can be on the same device!

A single computer can run:

- a web browser (client)

- a database server (server)

So, the same machine can be both at different times.

🤔 So why do we still call some computers “servers”?

Even though devices can do both roles, we name them based on their main purpose:

👉 If a machine mostly provides data → we call it a server

(Even if it sometimes acts like a client for something else.)

👉 If a machine mostly receives data → we call it a client

(Even if it occasionally sends something.)

Examples:

- “Email server” → serves email to clients

- “Desktop PC” → mostly requests data, so we call it a client device

📦 Simple Analogy

- Server = Restaurant kitchen → makes and provides food

- Client = Customer → requests and receives food

- But even the kitchen might “client” another service (like ordering ingredients).

⭐ Final Summary

| Term | Simple Meaning |
| --- | --- |
| Server | Provides data/services |
| Client | Requests data/services |
| Node | Any device on a network |
| A computer can be both | Yes! Many programs act as servers & clients |

🌐 What Is the Physical Layer? (Simple Explanation)

The physical layer is the bottom layer of networking.
It is responsible for actually sending 1s and 0s across cables or wireless signals.

Everything else in networking (frames, packets, protocols) depends on the physical layer working correctly.

🔢 What is a Bit?

- The tiniest piece of data a computer understands

- Can be only 1 or 0

Everything you do online—songs, videos, messages, bank transactions—is millions or billions of bits traveling across networks.

⚡ How do cables send 1s and 0s?

A copper cable carries a constant electrical charge.

To send data, devices change that voltage slightly.

This process is called:

👉 Modulation
(In networking, more specifically: line coding)

How it works (simple):

- Voltage Level A = 0

- Voltage Level B = 1

Both devices understand these voltage changes as bits.

🚀 How fast is this?

Modern Ethernet cables can send:

👉 10 billion bits per second
(10 Gbps)

All using tiny voltage shifts across wires thinner than a pencil.

🧠 Easy Analogy

Think of the physical layer as:

- Morse code operators sending dots and dashes

- But at lightning speed, billions of times per second

The patterns of electrical signals are the “code.”

🔍 What YOU need to understand (as an IT support specialist)

You do not need deep physics or math.

You just need to know:

- Cables carry electrical signals (or light, in fiber)

- These signals represent 1s and 0s

- Bad cables, broken connectors, interference → bad signals

- Bad signals → corrupted bits → failed communication

This is why troubleshooting often starts at the physical layer:

- Check cable

- Check connectors

- Check switch/router ports

- Ensure link lights are on

⭐ Final Simple Summary

- Physical layer = sends raw bits (1s and 0s)

- Copper uses voltage changes (modulation)

- Fiber uses light pulses

- Everything in networking depends on this layer

- Modern cables can transmit billions of bits per second

🧵 1. What is Twisted Pair Cable?

This is the most common type of network cable (Cat5e, Cat6).

Inside the cable:

- There are 8 copper wires

- They are grouped into 4 twisted pairs

Why are they twisted?

Twisting reduces:

- Electromagnetic interference

- Crosstalk (signals leaking between wires)

So the data stays clean.

🔄 2. How Duplex Communication Works

📌 Duplex = data can go BOTH ways

A network cable can send and receive data at the same time.

📌 Simplex = one-way only

(Like a TV broadcast: you only receive signals, not send.)

🚀 3. How do twisted pairs allow duplex?

Each twisted pair can be used for a direction of communication.

Example (simplified):

- Pair 1 → send data

- Pair 2 → receive data

OR in more advanced versions:

- Two pairs to send

- Two pairs to receive

Since there are multiple pairs, devices can talk simultaneously in both directions.

⭐ 4. Full Duplex vs Half Duplex

✔ Full Duplex

- Data flows both ways at the same time

- Modern networks ALWAYS use full duplex

- Faster, no waiting

Example:
Like a phone call — both people can talk at the same time.

❌ Half Duplex

- Data can flow in both directions

- BUT not at the same time

- If one device is sending, the other must wait

- Happens when there is a cable problem or device mismatch

Example:
Like a walkie-talkie — one talks, the other waits.

🛠️ 5. Why does a link sometimes fall back to half duplex?

If there is:

- Bad cable

- Damaged connector

- Wrong network setting

- Speed mismatch

- Poor hardware

Then the devices detect that full duplex isn’t stable, so they downgrade to half duplex for safety.

This causes:

- Slower speeds

- More collisions

- Poor performance

🎯 Final Simple Summary

| Concept | Simple Meaning |
| --- | --- |
| Twisted Pair | 4 pairs of twisted copper wires to reduce interference |
| Duplex | Both directions at once |
| Simplex | One-way only |
| Full Duplex | Normal networking today; simultaneous communication |
| Half Duplex | Slower; only one side speaks at a time |

🌐 1. RJ45 Plug (The Standard Network Plug)

- Twisted-pair Ethernet cables end with a connector called RJ45.

- This plug exposes the 8 small copper wires inside the cable.

- It clicks into an RJ45 network port on devices.

Devices that have RJ45 ports:

- Switches → many ports

- Servers & desktops → usually 1 or 2 ports

- Laptops/phones/tablets → usually none (use Wi-Fi or USB adapters)

🌈 2. What are Link Light and Activity Light?

Network ports usually have two tiny LED lights:

✔ Link Light

- Turns ON when:

  - A cable is plugged in

  - Both devices on each end have power

- If link light is OFF → no connection

✔ Activity Light

- Blinks when data is being transmitted

- Long ago, the blink matched every bit of data (1s/0s)

- Today networks are too fast →
The blink only shows “there is traffic”, not individual bits

On some switches:

- One LED may show BOTH link and activity

- Same LED might also show speed (e.g., green for 1 Gbps)

You must check the device manual for exact meaning.

🧱 3. Wall Ports & Patch Panels

Not every network port is on a device. Some are mounted:

- In walls

- Under desks

- In ceilings

These ports connect to cables running inside the building.

Where do these cables go?

They lead to a device called a patch panel, usually in a server room.

🧩 4. What is a Patch Panel?

A patch panel:

- Contains many RJ45 ports

- Does NOT switch or route anything

- It’s just an organized termination point for all building cables

Think of it like a big board of network jacks.

From the patch panel, short cables (called patch cables) connect to:

- Switches

- Routers

This gives network access to every wall port in the building.

🛠️ 5. Why this matters for IT troubleshooting

When diagnosing physical layer problems, always check:

- RJ45 plug connected properly?

- Link light ON?

  - If no → cable, port, or power problem.

- Activity light blinking?

  - If no → no traffic or possibly a deeper issue.

- Patch panel connections correct?

- Cable runs intact?

- Speed/duplex LEDs correct?

Understanding these basics helps solve many real-world network issues.

⭐ Final Simple Summary

| Component | Purpose | Easy Meaning |
| --- | --- | --- |
| RJ45 plug | Connects cable to device | Standard Ethernet connector |
| Network port | Receives RJ45 plug | Found on switches, PCs |
| Link LED | Shows connection status | ON = connected |
| Activity LED | Shows data movement | Blinks = traffic |
| Patch panel | Terminates wall cables | Organized connection board |

🌐 1. Why Ethernet Matters

Even though Wi-Fi and cellular are popular today, Ethernet is still the most common networking method in offices and data centers because it is:

- Fast

- Stable

- Reliable

Ethernet works at the Data Link Layer (Layer 2) and hides all physical details from the upper layers.
This means:

- Your browser does NOT care whether you use Wi-Fi or a cable

- The data link layer handles it

📦 2. Ethernet History (Simple)

- Invented around 1980

- Standardized in 1983

- Still used today with improvements (faster speeds, fiber, etc.)

Originally networks used shared cables, so ALL devices shared the same connection and collided easily.

Switches didn't exist yet.

💥 3. What is a Collision Domain? (Easy)

A collision domain is a network area where:

Only ONE device can talk at a time

If two devices transmit at once → collision → both must stop and retry.

This happened in old Ethernet because all devices were on one shared cable.

🟡 4. CSMA/CD — The Old Solution to Collisions

CSMA/CD = Carrier Sense Multiple Access with Collision Detection

Think of it like this:

🔹 Step 1 — Listen

“Is anyone talking on the line?”

🔹 Step 2 — If quiet → send data

No one talking → I send my packet.

🔹 Step 3 — Collision?

If two devices talk at the same time → signals collide.

🔹 Step 4 — Back off

Each device waits a random time before trying again.
(This prevents another collision.)

Today, switches eliminate collisions, so CSMA/CD is rarely used.

🆔 5. Why we need MAC Addresses

In a shared network (collision domain), EVERY device receives ALL data.

So, how does a device know if a message is meant for it?

👉 MAC address tells it.

A MAC Address:

- Is a 48-bit number

- Looks like: AA:BB:CC:DD:EE:FF

- Is globally unique (no two devices in the world share the same MAC)

Example format:

00:1A:2B:3C:4D:5E

🔢 6. Why 48-bit MAC addresses are unique

Total possible MAC addresses = 2⁴⁸ ≈ 281 trillion

That's why we don’t run out.

🏭 7. MAC Address Structure

A MAC address has two halves:

🧩 First 3 octets → OUI (Organizationally Unique Identifier)

Assigned to device manufacturers by IEEE.

Example:

- Apple has its own OUI

- Intel has its own OUI

- Cisco has its own OUI

So you can identify a device’s manufacturer just by looking at its MAC!

🧩 Last 3 octets → Assigned by the manufacturer

They must ensure each MAC they produce is unique.

📬 8. Why Ethernet uses MAC addresses

Ethernet frames include:

- Source MAC (sender)

- Destination MAC (receiver)

Even in a shared collision domain, devices ignore traffic not meant for them.

This ensures your computer only opens data addressed to YOUR MAC, not everyone else’s.

⭐ Simple Analogy

Think of a collision domain like a room full of people:

- Everyone can hear every message

- But each person only responds when they hear their own name

The name is the MAC address.

🎯 Final Summary Table

| Concept | Simple Explanation |
| --- | --- |
| Ethernet | Layer 2 protocol for wired networking |
| Collision Domain | Area where only one device can talk at a time |
| CSMA/CD | Old method to avoid collisions |
| MAC Address | Unique 48-bit hardware ID |
| OUI | First half of MAC → manufacturer |
| Switches | Remove collisions (modern Ethernet) |

🌐 1. Unicast (One-to-One)

👉 A message sent from one device to exactly one device.

Example:
Your laptop sending data directly to your printer.

How Ethernet knows it’s unicast:

- Look at the first octet (first 8 bits) of the destination MAC address.

- If the least significant bit (LSB) of that first octet is 0 → Unicast

Behavior:

- Frame is sent across the LAN

- Only the device with that exact MAC address accepts it

- All others ignore it

This is the most common type of communication.

🌐 2. Multicast (One-to-Many — but not everyone)

👉 A message sent to a group of devices, not just one.

Example:

- Live streaming

- Routing protocol updates

- Video conferencing groups

How Ethernet knows it’s multicast:

- If the LSB of the first octet of the destination MAC is 1 → Multicast

Behavior:

- Frame is delivered to all devices on the LAN

- Each device checks:

  - “Is this multicast group one I care about?”

  - If yes → accept

  - If no → ignore

Devices can be configured to “join” or “leave” multicast groups.

🌐 3. Broadcast (One-to-Everyone)

👉 A message sent to ALL devices on the LAN.

Example:

- ARP request ("Who has IP 192.168.1.5? Tell me!")

- Devices discovering each other

Broadcast MAC address:

FF:FF:FF:FF:FF:FF

This means:

- Every device must accept the frame

- No filtering

Broadcasts are useful for discovery but overuse can slow down a network.

⭐ Simple Comparison Table

| Type | Destination | MAC Address Pattern | Who receives it? |
| --- | --- | --- | --- |
| Unicast | One device | LSB of first octet = 0 | Only that one device |
| Multicast | A group | LSB of first octet = 1 | Devices in the group |
| Broadcast | Everyone | FF:FF:FF:FF:FF:FF | All devices |

🎯 Simple Analogy

Unicast → Sending a private letter to one person

Multicast → Sending a message to a WhatsApp group

Broadcast → Announcing something on a loudspeaker to everyone

🌐 What is an Ethernet Frame?

A frame is the name for a packet at the Ethernet (Layer 2) level.

It is a structured block of data that includes:

- Addresses

- Protocol information

- Actual data

- Error-checking bits

Every Ethernet frame follows the same basic format so devices can understand it.

🧱 The Structure of an Ethernet Frame (Simple Version)

Here are the parts, in the order they appear:

1️⃣ Preamble (8 bytes)

This is the "wake up, data is coming!" section.

- First 7 bytes = 10101010... (alternating 1s and 0s)

- Helps devices synchronize their clocks

- Makes sure both ends are ready for data

The 8th byte = Start Frame Delimiter (SFD)

This tells the receiving device:

👉 “Okay, the real frame starts now!”

2️⃣ Destination MAC Address (6 bytes)

Who the frame is meant for.

Example: AA:BB:CC:DD:EE:FF

3️⃣ Source MAC Address (6 bytes)

Who the frame came from.

4️⃣ EtherType or VLAN Header (2 bytes for EtherType)

This field tells which protocol is inside the frame.

For example:

- 0x0800 → IPv4

- 0x0806 → ARP

VLAN Tag (optional)

If present, it appears before EtherType.

VLANs allow:

- Multiple logical networks

- All running on the same physical switch

- Only devices in the same VLAN can talk to each other

Example VLAN uses:

- Phones on VLAN 10

- Desktops on VLAN 20

5️⃣ Payload (46 to 1500 bytes)

This is the actual data being transported.

Contains:

- IP header

- TCP/UDP header

- Application data

Minimum: 46 bytes
Maximum: 1500 bytes

(If data is too short, Ethernet adds padding.)

6️⃣ Frame Check Sequence (FCS) – 4 bytes

This is a checksum value used for error detection.

It is created using a method called CRC (Cyclic Redundancy Check).

🔍 How CRC Works (Simple Explanation)

Think of CRC like a “math fingerprint”:

- Sender runs the Ethernet frame through a mathematical formula

- It produces a number = checksum

- The checksum is added to the frame

- Receiver does the same math

- If both checksums match → data is good

- If not → data is corrupted → discard

Ethernet does NOT fix errors.
It only detects them.
Higher layers (like TCP) handle resending.

📦 Full Frame in Order (Summary)

- Preamble (7 bytes)

- Start Frame Delimiter (1 byte)

- Destination MAC (6 bytes)

- Source MAC (6 bytes)

- VLAN Tag (optional)

- EtherType (2 bytes)

- Payload (46–1500 bytes)

- Frame Check Sequence – CRC (4 bytes)

⭐ Super Simple Analogy

An Ethernet frame is like a shipping package:

| Frame Part | Shipping Analogy |
| --- | --- |
| Preamble | Truck honking before arriving |
| Destination MAC | Address on package |
| Source MAC | Return address |
| EtherType | Label saying what’s inside |
| VLAN Tag | Special handling instructions |
| Payload | The actual item in the box |
| CRC | Seal that shows if package was tampered |

🎉 Final Takeaway

You now understand:

- What an Ethernet frame is

- Every part inside the frame

- How VLANs fit in

- How CRC protects data integrity

This is a core networking skill used in IT support, cybersecurity, DevOps, cloud, and more.

1. What is an IP address?

- An IP address is like a house address for devices on the internet.

- It has 32 bits, split into 4 parts (called octets).

- Each part can be a number from 0 to 255.

  - Example of valid IP: 12.34.56.78

  - Invalid example: 123.456.789.100 (because 456 and 789 are impossible—numbers cannot exceed 255).

✅ 2. Why do we use dotted decimal notation?

- Instead of writing 32 bits (like 1100000010101000...), we write it in an easy form like 192.168.1.1.

- This format is called dotted decimal notation.

✅ 3. IP addresses are assigned to organizations

- Large companies get big blocks of IP addresses.

- Example: If IBM owns all IPs starting with 9, then:

  - Any address like 9.x.x.x belongs to IBM’s network.

So if a router sees 9.0.0.1, it only needs to send it to IBM’s network first.
IBM will deliver it internally from there.

This makes Internet routing faster and easier.

✅ 4. IP addresses belong to networks, not devices

- Your MAC address stays the same everywhere (hardware-based).

- Your IP address changes depending on where you connect:

  - At home → one IP

  - At a café → different IP

  - At office → another IP

Because every network gives you a new IP address.

✅ 5. How does your device get an IP address?

Usually, this happens automatically using a system called DHCP
(Dynamic Host Configuration Protocol).

DHCP does:

- Automatically gives an IP address when you connect to WiFi.

- You don't have to set anything manually.

This automatically assigned address = Dynamic IP.

✅ 6. Static vs Dynamic IP

Dynamic IP

- Given automatically by DHCP.

- Used for normal devices (phones, laptops, etc.).

- Changes from time to time.

Static IP

- Set manually.

- Stays the same forever.

- Used for:

  - Servers

  - Routers

  - CCTV systems

  - Printers

✅ What is an IP datagram?

- At the network layer, packets are called IP datagrams.

- Just like Ethernet uses frames, IP uses datagrams.

Each IP datagram has:

- Header → instructions about how to deliver the packet

- Payload → the real data being carried

✅ Important fields inside an IP datagram (explained simply)

1️⃣ Version (4 bits)

- Says whether it's IPv4 or IPv6.

2️⃣ Header Length (4 bits)

- Tells how big the header is.

- Usually 20 bytes for IPv4 (minimum size).

3️⃣ Service Type / QoS (8 bits)

- Used to mark some packets as “important”.
Example: voice calls > normal downloads.

4️⃣ Total Length (16 bits)

- Size of the entire datagram (header + data).

- Maximum is 65,535 bytes because 16 bits can represent that much.

✅ Fragmentation (breaking packets into pieces)

Why?

If a packet is too big to travel through a network with smaller limits, it must be split.

Fields used for fragmentation:

- Identification (16 bits) → marks packets that belong to the same message.

- Flags → tells if fragmentation is allowed.

- Fragment offset → tells how to reassemble pieces in correct order.

✅ TTL (Time to Live) – 8 bits

- A number that decreases by 1 every time the packet crosses a router.

- If TTL becomes 0, the packet is destroyed.

- Purpose: prevent packets from looping forever due to routing errors.

✅ Protocol field (8 bits)

Tells what transport layer protocol is inside:

- TCP

- UDP

Like saying: “the payload I’m carrying is a TCP segment”.

✅ Header Checksum

- Used for error checking of the header only.

- Must be recalculated at each router because TTL changes.

✅ Source & Destination IP (each 32 bits)

- The sender’s IP

- The receiver’s IP

Self-explanatory.

✅ Options + Padding

- Options: rarely used, for special testing.

- Padding: adds zeroes to keep the header size correct.

🧱 Layering (Encapsulation) – The big idea

This is the most important takeaway.

Ethernet frame

- Its payload = IP datagram

IP datagram

- Its payload = TCP/UDP segment

TCP/UDP segment

- Its payload = actual application data (like a message, file, video chunk)

This is why networking is taught in layers:
Each layer puts its entire packet inside the next layer.

⭐ Super Simple Analogy

Think of sending a gift:

| Layer | Analogy | Tech Term |
| --- | --- | --- |
| Application data | The gift | Data |
| TCP/UDP | The box around the gift | Segment |
| IP | The courier label | Datagram |
| Ethernet | The shipping bag | Frame |

The gift goes inside a box → the box goes inside a labeled package → the package goes inside a shipping bag.

That’s exactly how encapsulation works.

✅ 1. Every IP address has two parts

An IP address like 9.100.100.100 has:

- Network ID → tells which network the device is part of

- Host ID → tells which device inside that network

For example:
Class A example (IBM):

- IP: 9.100.100.100

- Network ID = 9

- Host ID = 100.100.100

✅ 2. Why do we need Address Classes?

Before CIDR existed, the world used address classes to split the global IP address space into three main types:

Class A → Very large networks

- First octet = Network ID

- Last 3 octets = Host ID

- Host space = 2²⁴ = 16,777,216 devices

Class B → Medium networks

- First 2 octets = Network ID

- Last 2 = Host ID

Class C → Small networks

- First 3 octets = Network ID

- Last octet = Host ID

- Host space = 2⁸ = 256 devices

✅ 3. How to identify Class A, B, C from the first octet

Instead of binary, you can look at the first number in the IP:

| Class | Binary Start | First Octet Range | Example |
| --- | --- | --- | --- |
| A | 0xxxxxxx | 0 – 127 | 9.3.5.1 |
| B | 10xxxxxx | 128 – 191 | 150.12.5.8 |
| C | 110xxxxx | 192 – 223 | 192.168.1.1 |

So:

- If an IP starts with 9, 45, 100, etc. → Class A

- If it starts with 150 or 172 → Class B

- If it starts with 192, 200, etc. → Class C

✅ 4. What about Class D and Class E?

Class D (224–239)

- Used for multicasting
(sending one packet to many devices at once)

Class E (240–255)

- Used for testing & research

- Not used for normal networks

✅ 5. Summary Table (super easy)

| Class | First Octet | Usage | Host Capacity |
| --- | --- | --- | --- |
| A | 0–127 | Huge networks (IBM, big orgs) | ~16 million |
| B | 128–191 | Medium networks | ~65k |
| C | 192–223 | Small networks | 256 |
| D | 224–239 | Multicast | — |
| E | 240–255 | Experimental | — |

⭐ Ultra-simple explanation

Think of IP classes like sizes of apartment buildings:

- Class A = mega city (millions of houses)

- Class B = big town

- Class C = small colony

- Class D = broadcast to everyone

- Class E = under construction, not used

✅ What is ARP?

ARP (Address Resolution Protocol) is a helper system that finds the MAC address when you only know the IP address.

Because:

- IP address = used at Network Layer

- MAC address = needed to actually send data at Data Link Layer

So before sending data, your device must find the destination MAC address.

✅ Why do we need ARP?

When your computer wants to send something:

- It knows the IP address of the target.

- But it cannot send an Ethernet frame without the MAC address.

So it must ask:
➡️ “Who has this IP? Tell me your MAC address!”

That is exactly what ARP does.

✅ How ARP works (very simple steps)

Step 1 — Check ARP Table

Your computer first checks its ARP table (a small list that maps IP → MAC).

Example entry:
10.20.30.40 = AA:BB:CC:DD:EE:11

If the MAC address is already saved → no need to do anything.

Step 2 — If MAC not found → Send ARP Broadcast

If the entry is missing:

Your computer sends a broadcast message to everyone:

“Who has IP 10.20.30.40? Please reply with your MAC address.”

Broadcast MAC address = FF:FF:FF:FF:FF:FF
This reaches every device in the local network.

Step 3 — Correct device replies

The device with IP 10.20.30.40 replies:

“I have that IP. My MAC address is AA:BB:CC:DD:EE:11.”

Step 4 — Your computer stores the result

- It saves this mapping in the ARP table for future use.

- So next time, it doesn’t need to ask again.

Example saved entry:

| IP | MAC |
| --- | --- |
| 10.20.30.40 | AA:BB:CC:DD:EE:11 |

Step 5 — Now Ethernet frame can be sent

Your device now knows the destination MAC, so it can build the Ethernet frame and send the data.

✅ Why ARP entries expire?

Networks change — devices move, IPs change.
So ARP entries delete automatically after a short time to avoid outdated information.

⭐ Super Simple Analogy

Think of IP = house number
MAC = person’s physical name

You know the house number (IP) but need the person’s name (MAC) to deliver a letter.

So you ask the entire neighbourhood:

“Who lives at house 10.20.30.40?”

The correct person replies:

“I do! My name is AA:BB:CC...”

Then you deliver the letter → and save the info for next time.

Big picture first 🌍

The Internet is not one big network.
It is millions of small networks connected together.

👉 Routing is the process that decides which path your data takes to reach its destination.

Think of routing like Google Maps for data.

What is a router? 🤔

A router is like a traffic police officer for the Internet.

- It connects two or more networks

- It receives data

- It checks where the data should go

- It forwards it in the right direction

A router must have at least 2 connections (otherwise it can’t route).

Simple routing in 4 steps 🚦

Every router does the same basic thing:

- Receives a packet

- Looks at the destination IP address

- Checks its routing table (a list of where networks are)

- Forwards the packet through the best interface

This repeats until the packet reaches its destination.

Important idea to remember 🔑

👉 IP addresses belong to networks, not to cables or MAC addresses

- IP address = logical location (like a city name)

- MAC address = physical identity (like a house number)

Example 1: Two networks (easy case) 🧩

Networks:

- Network A → 192.168.1.0/24

- Network B → 10.0.0.0/24

Router:

- On Network A → 192.168.1.1

- On Network B → 10.0.0.254

Computer:

- PC on Network A → 192.168.1.100

- Wants to talk to → 10.0.0.10

What happens step-by-step 📦

- The computer checks:

  - “Is 10.0.0.10 in my network?”

  - ❌ No

- So it sends the packet to its gateway (router)
👉 using the router’s MAC address

- The router:

  - Removes the old Ethernet wrapping

  - Looks at the destination IP → 10.0.0.10

- Router checks its routing table:

  - “Ah! 10.0.0.0/24 is directly connected”

- Router:

  - Reduces TTL by 1 (to avoid infinite loops)

  - Wraps the packet again

  - Sends it to 10.0.0.10

✅ Data delivered!

Example 2: Three networks (realistic case) 🌐

Now add:

- Network C → 172.16.1.0/23

- Second router connects Network B ↔ Network C

Destination:

- From 192.168.1.100

- To 172.16.1.100

What happens now? 🔁

- PC sends data to its gateway (Router 1)

- Router 1:

  - Sees destination is not directly connected

  - Knows another router (10.0.0.1) leads there

  - Forwards packet

- Router 2:

  - Sees 172.16.1.100 is on its network

  - Sends packet directly to the destination

✅ Packet reaches final computer

What changes on the Internet? 🌍

Only one thing: scale

- Not 2–3 routers

- But 10, 15, 20+ routers

- Many possible paths (mesh network)

Routers always:

- Look at destination IP

- Check routing table

- Send packet closer to destination

Why TTL exists ⏳

TTL = Time To Live

- Prevents packets from looping forever

- Reduced by 1 at every router

- When it reaches 0 → packet is dropped

One-line summary 💡

Routing is the process where routers read the destination IP address and forward packets step-by-step across networks until they reach the correct computer.

What is a routing table? 🗂️

A routing table is like a route chart or map inside a router.

👉 It tells the router:

- Where a network is

- Which direction to send data

- Which path is best

Every router checks this table before sending any packet.

Important thing to know 🔑

Routing itself is simple:

Look at destination → check table → send in right direction

The routing table is what makes this possible.

Old routers vs today’s routers 🕰️

- Early routers were just normal computers

- They had:

  - 2 network cards

  - Manually filled routing tables

- Even today:

  - Your laptop, phone, Linux, Windows all have routing tables

  - They decide whether data goes:

    - To your local network

    - Or to your router (Internet)

👉 You could still make a router today with:

- A PC

- Two network interfaces

- A routing table

What’s inside a routing table? 📋

A basic routing table has 4 main columns.

Let’s understand each very simply.

1️⃣ Destination Network 🎯

This column answers:

“Which network is this data meant for?”

Example:

- 192.168.1.0/24

- 10.0.0.0/8

This tells the router:

- “Any IP that starts like this belongs to this network”

🧠 When a packet arrives:

- Router looks at the destination IP

- Matches it to a network in this column

Default / catch-all entry 🌐

If the router doesn’t recognize the destination network:

- It uses a default route

- Often written as:

  - 0.0.0.0/0

👉 Meaning:

“Send it this way if I don’t know exactly where it belongs”

2️⃣ Next Hop ➡️

This answers:

“Who should I send this packet to next?”

It can be:

- Another router’s IP address

- Or directly connected (no router in between)

Example:

- Next hop = 10.0.0.1

- Or: “Directly connected”

3️⃣ Total Hops (Distance) 📏

This tells:

“How far away is this network?”

- A hop = passing through one router

- Fewer hops = shorter path

Why this matters:

- There are many paths on the Internet

- Routers prefer:

  - Faster paths

  - Shorter paths

But paths can change because:

- Routers go down

- Cables break

- Traffic gets heavy

- New routes appear

So routers:

- Keep updating distances

- Always try to use the best path

4️⃣ Interface 🔌

This answers:

“Which port should I send the data out from?”

Routers have multiple interfaces:

- Ethernet

- Fiber

- WAN

- LAN

The routing table tells:

- Exactly which interface to use for each destination network

How a router uses the table (simple flow) 🔁

- Packet arrives

- Router reads destination IP

- Looks for matching destination network

- Picks:

  - Best next hop

  - Fewest hops

- Sends packet out the correct interface

This happens for every single packet.

Why this is impressive 🤯

- Home routers → few routes

- Core Internet routers → millions of routes

- And they:

  - Check the routing table

  - For every packet

  - In microseconds

That’s how the Internet stays fast.

One-line summary 💡

A routing table is a map inside a router that tells it where to send data, who to send it to next, how far the destination is, and which interface to use.

Big Picture (What is this topic about?)

Routers need to decide which path is fastest to send data.

To do this, routers:

- Keep a routing table (like a map)

- Talk to other routers

- Update their maps automatically when a better path is found

The “talking” between routers happens using routing protocols.

1️⃣ What are Routing Protocols?

👉 Routing protocols are rules that routers use to talk to each other.

Think of routers as people asking:

“Hey, do you know a faster way to reach this place?”

By sharing information:

- Routers learn new routes

- They learn shorter / faster paths

- Even routers very far away can learn about each other

That’s how the Internet stays fast 🌍⚡

2️⃣ Two Big Categories of Routing Protocols

🔹 Interior Gateway Protocols (IGP)

- Used inside one organization

- Example:

  - A company with many offices

  - An Internet Service Provider (ISP) within one country

👉 This is the main focus of your text.

🔹 Exterior Gateway Protocols (EGP)

- Used between different organizations

- Example:

  - Google’s network talking to Airtel’s network

(We don’t go deep into this here.)

3️⃣ What is an Autonomous System? (Very Important)

An autonomous system =
A group of networks controlled by one owner

Examples:

- A large company

- An ISP like Jio, Airtel, BSNL

All routers inside this group use Interior Gateway Protocols.

4️⃣ Types of Interior Gateway Protocols

There are two main types:

- Distance Vector Protocols (older, simpler)

- Link State Protocols (newer, smarter)

Let’s understand both.

5️⃣ Distance Vector Protocols (Simple Explanation)

🧠 How they think:

“I only know how far networks are from me.”

Each router:

- Keeps a list of networks

- Stores number of hops (router-to-router jumps)

🗣️ How they communicate:

- A router sends its entire routing table to its neighbors

- Neighbors update their own tables if they find a shorter path

📦 Example (Router A, B, C)

- Router A thinks:

  - Network X is 4 hops away via Router C

- Router B knows:

  - Network X is 2 hops away

Router B tells Router A:

“Hey, I can reach Network X in 2 hops.”

Router A thinks:

- 1 hop to Router B + 2 hops = 3 hops

- That’s better than 4 hops!

👉 Router A updates its routing table
👉 Sends traffic through Router B instead of Router C

❌ Problems with Distance Vector

- Routers only know about neighbors

- They don’t see the full network

- If something breaks far away, they are slow to notice

That’s why a better method was needed.

6️⃣ Link State Protocols (Smarter Way)

🧠 How they think:

“I want to know everything about the network.”

Each router:

- Shares the state of all its links

- Tells others:

  - Who it’s connected to

  - Whether links are up or down

  - How fast they are

🌐 What happens next?

- This information is shared with all routers

- Every router builds a full map of the network

- Each router calculates:

  - “What is the best path to every destination?”

This is like Google Maps for routers 🗺️

💻 Why Link State is Better

✅ Faster reaction to failures
✅ More accurate paths
✅ Better for large networks

❌ Needs more:

- Memory

- CPU power

But modern routers are powerful, so this is not a problem anymore.

7️⃣ Final Simple Comparison

| Feature | Distance Vector | Link State |
| --- | --- | --- |
| Knowledge | Only neighbors | Full network |
| Speed | Slow updates | Fast updates |
| Complexity | Simple | Complex |
| Modern usage | Mostly outdated | Widely used |

One-Line Summary 💡

Distance vector protocols share distance information with neighbors, while link state protocols share full network information so every router can calculate the best path on its own.

Top of Form

Bottom of Form